BDC-4000 M&C Update Archive File

FW/11808

Table of Contents

Version 1.0.0    FW/11808- Initial Release.

Dated: 07/19/05

Version 1.1.1 (FW/11808A)

Dated: 08/12/05

1) Added user remote command for emulating synthesizer fault.

Syntax: FTS=x, where x=0(normal) or 1(faulted).

See flag <V:A-1>. Files affected: cinit.c, global_var.h, remote.c

Version 1.1.2 (FW/11808B)

Dated: 1/3/06

1) Added nv_frequency update in the FRE_serv function and a Calculate_LO_Freq so that if you power down after you change the frequency it will update.

See flag <V:B-1>. Files affected: main.c remote.c

2) Added a memory peek/poke utility that allows you to read and write memory

locations for debug.

See flag <V:B-2>. Files affected: remote.c

Version 1.1.3 (FW/11808C)

Dated: 2/14/06

1) Fixed hieroglyphics at the end of the CCS command by

Hard-coding the "XXXX,X" into the sprintf command in the remote.c

file in the CCS_serv function.

See flag <V:C-3>. Files affected: remote.c

2) Removed AFR capability. According to Brad, it's not used by the

customer when installed as a system with the MBT base unit.

See flag <V:C-1>. Files affected: cfaults.c

3) Fixed intermittent muting during power-up. This caused by improper

mute handling in relation to fault condition.

See flag <V:C-2>. Files affected: main.c, cinit.c, cfaults.c, global.h,

remote.c, bgc.c

Version 1.1.4 (FW/11808D)

Dated: 10/17/06

1) 9/26/2006

Made EOD show hours instead of minutes

See flag <V:D-1>.

Files affected: remote.c

2) 10/17/2006

Get rid of redundancy because it was messing up the LNAC reading

See flag <V:D-2>.

Files affected: remote.c, main.c

3) 10/18/2006

Fixed LNAC Current reading

See flag <V:D-3>.

Files affected: control.c, main.c

Version 1.1.5(FW/11808F)

1) Padded the LNA current value for GSI ,

made a new nv variable <V:F-1>

2) 1/07/2007

Changed the FRW and CID commands so they

return an "=" when queried   <V:F-2>     Files affected: remote.c

3) 1/25/2007

Fixed the bad numbers at the end of the CCS command

<V:F-3>     Files affected: remote.c