CiM25/2020D 1.0.4 Release

| Approval Signatures |  |
| --- | --- |
| CiM25 Product Manager: Date: Jeff Harig | CiM25/2020D Originator: Date: Yunsong Wang |

Applicability

System upgrade for CiM25/2020D Release 1.0.1, 1.0.2, 1.0.3 Systems

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Rev # | Description | Date | Prepared by |
| - | Initial Release | 11/08/2002 | Yunsong Wang |

FSCM No. 4J515

Table of Contents

Introduction	3

Features	3

CiM25 Firmware/Software Valid Configurations	3

Issues Addressed In This Release	3

Known Issue(s) In This Release	4

# Introduction

The primary purpose of the CiM25/2020D Version 1.0.4 Release is to fix security problem with user login and restruture MIB file to be consistent across product lines.

To use SNMP interface, the new CiM25 MIB file and SDM2020D MIB file is required for this release.

# Features

- Restructured MIB file to be consistent across product lines. Separated cim25 mib and sdm2020D mib to two files. Added descriptions and moved fault trap in MIB files.

- Implemented EndOfMibView in SNMP.

- Added Trap Community entry to CiM25 MIB, HTTP interface and Telnet interface.

# CiM25 Firmware/Software Valid Configurations

The table below identifies the valid configurations of CiM25/2020D Firmware, CiM25 MIB, and SDM2020D MIB which are supported. CiM25/2020D that are configured with software that does not match one of the entries below may not function as expected.

| Release # | CiM25/2020D Firmware | CiM25 MIB | SDM2020D MIB | Combined MIB (Cim25_2020D) |
| --- | --- | --- | --- | --- |
| 1.0.1 | F1623-1-.ccc |  |  | F1623-2-.MY |
| 1.0.2 | F1623-1A.ccc |  |  | F1623-2A.MY |
| 1.0.3 | F1623-1B.ccc |  |  | F1623-2B.MY |
| 1.0.4 | F1623-1C.ccc | F1630-1A.MY | F1623-3-.MY |  |

# Issues Addressed In This Release

- Fixed the incorrect display of the Support Email Subject.

- Fixed Trap Enterprise response in trap message.

- Fixed GetBulk command in SNMP.

- Fixed telnet problem of sending empty command to modem if enter is pressed.

- Fixed problem with extra htm text on web page after Support page being submited.

- Fixed problem with username and password check.

- Fixed improper GET response in MIB Browser for MIB item 'submitconfig' and 'rxBufferCenter'.

- Fixed the problem that CiM25 didn’t trap on Common Faults of SDM2020D modem.

# Known Issue(s) In This Release

Issue:

On Utilities and Clocks web page, the status bar at the bottom of the web browser doesn’t disappear even when the loading of the web page content is complete. This happens on some Internet Explorer, however, it doen’t affect the functionality of CiM25 web interface.

Workaround:

None currently